VIDEO PIPELINE

Step 1 — Getting video out of the DVR

Hikvision DVRs expose streams in two standard ways. RTSP (Real Time Streaming Protocol) is the most universal — every Hikvision DVR has an RTSP URL per camera channel, something like rtsp://admin:password@dvr-ip:554/Streaming/Channels/101. ONVIF is the other option, an industry standard that lets your backend discover and control the DVR without Hikvision-specific code. You can also use Hikvision's own ISAPI (HTTP-based REST API) to pull clips, snapshots, and event data.

Step 2 — The backend is the critical middle layer

Your React dashboard cannot directly consume an RTSP stream — browsers don't support RTSP natively. You need a backend server that:

- Pulls the RTSP stream from the DVR using FFmpeg

- Transcodes it to HLS (HTTP Live Streaming) or WebRTC, which browsers understand natively

- Runs your object detection model (YOLO, TensorFlow, or similar) on the frames

- Pushes detection events and annotated frames to your React frontend via WebSocket or REST API

FFmpeg is the standard tool for the RTSP-to-HLS conversion. A typical command pulls the stream and chunks it into HLS segments your React app can play with something like hls.js or video.js.

Step 3 — React receives clean, usable data

Your React dashboard then gets a video stream it can actually render, plus a separate WebSocket feed of detection events (person detected, vehicle entered zone, etc.) that you overlay as annotations, trigger alerts from, and write to your database for reporting.